CN Lab Exam – Q8 to Q15 (Cisco Packet Tracer)

Pranathi Bandi · 24BCE2079 · all files built and tested in Packet Tracer 9.0

# 1. Links to the files

From anywhere: onedrive.com → sign in → My files → Desktop → CN_Lab_Exam → download the .pkt → open in Packet Tracer.

The links below are OneDrive view-only links – they open from any phone or computer (Ctrl + click in Word). On the page that opens, click Download, then open the .pkt in Packet Tracer.

| Q | Experiment | File (Ctrl + click) |
| --- | --- | --- |
| Q8 | IP addressing and default gateway | Q8_IP_Gateway.pkt |
| Q9 | RIP | Q9_RIP.pkt |
| Q9 | OSPF | Q9_OSPF.pkt |
| Q10 | SMTP and DNS | Q10_SMTP_DNS.pkt |
| Q11 | Unicast routing – LSR (OSPF) | Q11_LSR_OSPF.pkt |
| Q12 | Unicast routing – DVR (RIP) | Q12_DVR_RIP.pkt |
| Q13 | Unicast routing – BGP | Q13_BGP.pkt |
| Q14 | TCP and UDP traffic control | Q14_TCP_UDP.pkt |
| Q14 | Python script (Programming tab) | Q14_code.py |
| Q15 | DNS client–server | Q15_DNS_Client_Server.pkt |

C programs Q1–Q7 (open on this laptop): q1_parity.c · q2_checksum.c · q3_crc.c · q4_hamming.c · q5_stopwait.c · q6_gobackn.c · q7_selective.c

# 2. Rules that make every question work

- Router: 2911. Cable: orange Automatic lightning.

- Router CLI start: type "no", Enter, Enter → en → conf t → no ip domain-lookup.

- After every ip add, type no sh.

- Every PC behind a router needs a Default Gateway (the router’s .1).

- Test: envelope ✉ (Add Simple PDU) → click PC A → click PC B → bottom-right says Successful. If the first one says Failed, send it again (ARP).

- Speed up: click ⏩ next to the clock a few times (green lights, RIP/OSPF/BGP convergence).

# Q8 – IP addressing and default gateway

File: Q8_IP_Gateway.pkt

Build: 1 router (2911), 2 switches (2960), 3 PCs on each switch. Connect Router → Switch0 first (becomes g0/0), then Router → Switch1 (g0/1).

| Device | IP address | Subnet mask | Default gateway |
| --- | --- | --- | --- |
| Router g0/0 | 192.168.1.1 | 255.255.255.0 | — |
| Router g0/1 | 192.168.2.1 | 255.255.255.0 | — |
| PC0 / PC1 / PC2 | 192.168.1.2 / .3 / .4 | 255.255.255.0 | 192.168.1.1 |
| PC3 / PC4 / PC5 | 192.168.2.2 / .3 / .4 | 255.255.255.0 | 192.168.2.1 |

en

conf t

no ip domain-lookup

int g0/0

ip add 192.168.1.1 255.255.255.0

no sh

int g0/1

ip add 192.168.2.1 255.255.255.0

no sh

end

wr

Result: PC0 → PC3 and PC5 → PC1 = Successful.

# Common network for Q9, Q11, Q12, Q13

PC0, PC1 ─ Switch0 ─ Router0 ═══ Router1 ─ Switch1 ─ PC2, PC3

Cable order: Router0 → Switch0, Router1 → Switch1, then Router0 → Router1 (so the router link is g0/1 on both), then PCs → switches.

| Device | IP address | Gateway |
| --- | --- | --- |
| PC0 / PC1 | 192.168.1.2 / 192.168.1.3 | 192.168.1.1 |
| Router0 g0/0 (LAN) | 192.168.1.1 | — |
| Router0 g0/1 (to Router1) | 10.0.0.1 | — |
| Router1 g0/1 (to Router0) | 10.0.0.2 | — |
| Router1 g0/0 (LAN) | 192.168.2.1 | — |
| PC2 / PC3 | 192.168.2.2 / 192.168.2.3 | 192.168.2.1 |

Router0 base:

en

conf t

no ip domain-lookup

int g0/0

ip add 192.168.1.1 255.255.255.0

no sh

int g0/1

ip add 10.0.0.1 255.255.255.0

no sh

exit

Router1 base:

en

conf t

no ip domain-lookup

int g0/0

ip add 192.168.2.1 255.255.255.0

no sh

int g0/1

ip add 10.0.0.2 255.255.255.0

no sh

exit

# Q9 – RIP and OSPF

RIP file: Q9_RIP.pkt

OSPF file: Q9_OSPF.pkt

## RIP

Router0:

router rip

version 2

no auto-summary

network 192.168.1.0

network 10.0.0.0

end

wr

Router1:

router rip

version 2

no auto-summary

network 192.168.2.0

network 10.0.0.0

end

wr

Check: sh ip route → R 192.168.2.0 [120/1] via 10.0.0.2. PC0 → PC2 = Successful.

## OSPF

Router0:

router ospf 1

network 192.168.1.0 0.0.0.255 area 0

network 10.0.0.0 0.0.0.255 area 0

end

wr

Router1:

router ospf 1

network 192.168.2.0 0.0.0.255 area 0

network 10.0.0.0 0.0.0.255 area 0

end

wr

Check: log shows OSPF … LOADING to FULL; sh ip route → O 192.168.1.0 [110/2]. PC1 → PC3 = Successful.

Tip: to switch an existing file from RIP to OSPF: conf t → no router rip → then the OSPF lines.

# Q10 – SMTP and DNS

File: Q10_SMTP_DNS.pkt

Build: Server0 + Switch0 + PC0 + PC1, all on the switch. No router, no gateway.

| Device | IP address | DNS server |
| --- | --- | --- |
| Server0 | 192.168.1.10 | — |
| PC0 | 192.168.1.2 | 192.168.1.10 |
| PC1 | 192.168.1.3 | 192.168.1.10 |

- Server0 → Services → DNS → On. Add A Records: www.cisco.com → 192.168.1.10, mail.cisco.com → 192.168.1.10, pc1.cisco.com → 192.168.1.3.

- Server0 → Services → EMAIL → SMTP ON, POP3 ON → Domain cisco.com → Set → add users pc0 / 123 and pc1 / 123 with “+”.

- PC0 → Desktop → Email: name pc0, email pc0@cisco.com, incoming + outgoing mail.cisco.com, user pc0, password 123 → Save.

- PC1 → same with pc1 / pc1@cisco.com.

- PC0 → Compose → To pc1@cisco.com, Subject 24bce2079 → Send → “Send Success”.

- PC1 → Receive → “Receive Mail Success” and the mail appears.

# Q11 – Unicast routing with LSR

File: Q11_LSR_OSPF.pkt

LSR in Packet Tracer = OSPF. Use the common network + the OSPF commands from Q9.

Viva: Link-state: every router knows the whole map and runs Dijkstra’s shortest-path algorithm. Metric = cost (bandwidth).

# Q12 – Unicast routing with DVR

File: Q12_DVR_RIP.pkt

DVR in Packet Tracer = RIP. Use the common network + the RIP commands from Q9.

Viva: Distance-vector: routers share tables with neighbours every 30 s using Bellman-Ford. Metric = hop count, max 15. Problem: count-to-infinity (fixed by split horizon).

# Q13 – Unicast routing with BGP

File: Q13_BGP.pkt

Common network, no RIP/OSPF (if present: no router rip / no router ospf 1).

Router0 (AS 100):

conf t

router bgp 100

neighbor 10.0.0.2 remote-as 200

network 192.168.1.0 mask 255.255.255.0

end

wr

Router1 (AS 200):

conf t

router bgp 200

neighbor 10.0.0.1 remote-as 100

network 192.168.2.0 mask 255.255.255.0

end

wr

Check: “%BGP-5-ADJCHANGE: neighbor … Up”; press ⏩ ~8 times; sh ip route → B 192.168.1.0 [20/0]. PC0 → PC3 = Successful.

# Q14 – TCP and UDP traffic control

File: Q14_TCP_UDP.pkt

Code: Q14_code.py

Build: PC0 + PC1 + Switch0. PC0 = 192.168.1.1, PC1 = 192.168.1.2, mask 255.255.255.0.

- PC0 → Programming → New → name ex8, template Empty - Python3 → Create → open main.py → paste Q14_code.py → keep ROLE = "server" → Run.

- PC0 output: [TCP] server listening on port 5000: True and [UDP] server listening on port 6000: True.

- PC1 → same, but change line 11 to ROLE = "client" → Run. Wait ~40 seconds.

- Result: TCP rounds (slow start, flow control, congestion, Go-Back-N) and UDP retransmissions, both ending RESULT: SUCCESS - 100% delivered.

- When reopening the saved file: click Run on PC0 first, then on PC1.

Backup (no Programming tab): Server with HTTP + DNS, PC browses www.cisco.com in Simulation mode with filters TCP, UDP, DNS, HTTP → show the TCP 3-way handshake vs UDP with no handshake.

# Q15 – DNS client–server

File: Q15_DNS_Client_Server.pkt

Same build and IPs as Q10 (Server0 192.168.1.10, PC0 .2, PC1 .3, both DNS server 192.168.1.10). DNS records: www.cisco.com → 192.168.1.10, pc1.cisco.com → 192.168.1.3.

Test on PC0 → Command Prompt:

nslookup www.cisco.com     -> Address: 192.168.1.10

nslookup pc1.cisco.com     -> Address: 192.168.1.3

ping www.cisco.com         -> Reply from 192.168.1.10 (4/4, 0% loss)

Also: PC0 → Web Browser → www.cisco.com → page opens.

# If anything says Failed

- Red dot on a cable → go into that router port and type no sh.

- Check IP, mask, gateway (and DNS server for Q10/Q15) against the tables.

- Press ⏩ a few times and send the envelope again – the first one often fails.

- Router window won’t take typing → Window menu → pick the router; or close and reopen it.

- Still stuck → Simulation mode → Play ▶ – the ❌ shows exactly where it stops.