CN Lab Exam – Final (Q1 to Q15)

Pranathi Bandi · 24BCE2079 · all files built and tested in Packet Tracer 9.0

# 1. Links to the files

GitHub (everything): https://github.com/pranathibandi37-cyber/cn_lab → click a file → Download raw file.

From anywhere: onedrive.com → sign in → My files → Desktop → CN_Lab_Exam → download the .pkt → open in Packet Tracer.

The links below are OneDrive view-only links – they open from any phone or computer (Ctrl + click in Word). On the page that opens, click Download, then open the .pkt in Packet Tracer.

| Q | Experiment | File (Ctrl + click) |
| --- | --- | --- |
| Q8 | IP addressing and default gateway | Q8_IP_Gateway.pkt |
| Q9 | RIP | Q9_RIP.pkt |
| Q9 | OSPF | Q9_OSPF.pkt |
| Q10 | SMTP and DNS | Q10_SMTP_DNS.pkt |
| Q11 | Unicast routing – LSR (OSPF) | Q11_LSR_OSPF.pkt |
| Q12 | Unicast routing – DVR (RIP) | Q12_DVR_RIP.pkt |
| Q13 | Unicast routing – BGP | Q13_BGP.pkt |
| Q14 | TCP and UDP traffic control | Q14_TCP_UDP.pkt |
| Q14 | Python script (Programming tab) | Q14_code.py |
| Q15 | DNS client–server | Q15_DNS_Client_Server.pkt |

C programs Q1–Q7 (GitHub): q1_parity.c · q2_checksum.c · q3_crc.c · q4_hamming.c · q5_stopwait.c · q6_gobackn.c · q7_selective.c

# 2. Rules that make every question work

- Router: 2911. Cable: orange Automatic lightning.

- Router CLI start: type "no", Enter, Enter → en → conf t → no ip domain-lookup.

- After every ip add, type no sh.

- Every PC behind a router needs a Default Gateway (the router’s .1).

- Test: envelope ✉ (Add Simple PDU) → click PC A → click PC B → bottom-right says Successful. If the first one says Failed, send it again (ARP).

- Speed up: click ⏩ next to the clock a few times (green lights, RIP/OSPF/BGP convergence).

# PART A – C programs (Q1–Q7)

Code::Blocks → File → New → Empty file → paste → Save as qN.c (must end in .c) → press F9. Type the inputs shown. All 7 were compiled and tested.

## Q1 – Error detection using even and odd parity

File: q1_parity.c

// Q1: Error detection using even and odd parity check

#include <stdio.h>

int main(){

char data[50], recv[51];

int i, ones = 0, even, odd;

printf("Enter binary data: ");

scanf("%s", data);

for(i = 0; data[i]; i++)

if(data[i] == '1') ones++;

even = ones % 2;

odd = !even;

printf("Number of 1s = %d\n", ones);

printf("Even parity bit = %d -> Transmitted: %s%d\n", even, data, even);

printf("Odd parity bit  = %d -> Transmitted: %s%d\n", odd, data, odd);

printf("\nEnter received data (with parity bit): ");

scanf("%s", recv);

ones = 0;

for(i = 0; recv[i]; i++)

if(recv[i] == '1') ones++;

printf("Even parity check: %s\n", ones % 2 == 0 ? "No error detected" : "Error detected");

printf("Odd parity check : %s\n", ones % 2 == 1 ? "No error detected" : "Error detected");

return 0;

}

How to run: Input: 1011001 then 10110010 → even parity bit 0, “No error detected”.

## Q2 – Error detection using checksum

File: q2_checksum.c

// Q2: Error detection using Checksum (8-bit words, 1's complement)

#include <stdio.h>

#include <stdlib.h>

void printbin(int x){

int i;

for(i = 7; i >= 0; i--) printf("%d", (x >> i) & 1);

}

int main(){

int n, i, sum = 0, cs;

char w[20];

printf("Enter number of data words: ");

scanf("%d", &n);

printf("Enter %d words (8 bits each):\n", n);

for(i = 0; i < n; i++){

scanf("%s", w);

sum += strtol(w, NULL, 2);

if(sum > 255) sum = (sum & 255) + 1;    // wrap-around carry

}

cs = ~sum & 255;

printf("Sum      = "); printbin(sum);

printf("\nChecksum = "); printbin(cs);

printf("\n");

printf("\nReceiver: enter the %d received words:\n", n);

sum = 0;

for(i = 0; i < n; i++){

scanf("%s", w);

sum += strtol(w, NULL, 2);

if(sum > 255) sum = (sum & 255) + 1;

}

sum += cs;

if(sum > 255) sum = (sum & 255) + 1;

printf("Receiver sum = "); printbin(sum);

printf("\n%s\n", sum == 255 ? "No error detected" : "Error detected");

return 0;

}

How to run: Input: 2, 10110011, 01011100, then the same two words again → checksum 11101111, “No error detected”.

## Q3 – Error detection using CRC

File: q3_crc.c

// Q3: Error detection using Cyclic Redundancy Check (CRC)

#include <stdio.h>

#include <string.h>

void divide(char t[], char g[]){

int i, j, n = strlen(t), k = strlen(g);

for(i = 0; i <= n - k; i++)

if(t[i] == '1')

for(j = 0; j < k; j++)

t[i + j] = (t[i + j] == g[j]) ? '0' : '1';

}

int main(){

char msg[50], gen[20], t[70], code[70];

int i, m, k, err = 0;

printf("Enter the binary message: ");

scanf("%s", msg);

printf("Enter the generator: ");

scanf("%s", gen);

m = strlen(msg);

k = strlen(gen);

strcpy(t, msg);

for(i = 0; i < k - 1; i++) t[m + i] = '0';

t[m + k - 1] = '\0';

divide(t, gen);

strcpy(code, msg);

strcat(code, t + m);

printf("CRC Remainder     : %s\n", t + m);

printf("Transmitted Frame : %s\n", code);

printf("\nEnter received frame: ");

scanf("%s", t);

divide(t, gen);

printf("Receiver remainder: %s\n", t + strlen(t) - (k - 1));

for(i = strlen(t) - (k - 1); t[i]; i++)

if(t[i] == '1') err = 1;

printf("%s\n", err ? "Error detected" : "No error detected");

return 0;

}

How to run: Input: 1001, 1011, 1001110 → remainder 110, frame 1001110, “No error detected”.

## Q4 – Error detection and correction using Hamming code

File: q4_hamming.c

// Q4: Error detection and correction using Hamming Code (7,4)

#include <stdio.h>

int main(){

int h[8], i, c1, c2, c4, pos;

printf("Enter 4 data bits (space separated): ");

scanf("%d %d %d %d", &h[3], &h[5], &h[6], &h[7]);

h[1] = h[3] ^ h[5] ^ h[7];

h[2] = h[3] ^ h[6] ^ h[7];

h[4] = h[5] ^ h[6] ^ h[7];

printf("Parity bits: P1=%d P2=%d P4=%d\n", h[1], h[2], h[4]);

printf("Hamming code: ");

for(i = 1; i <= 7; i++) printf("%d", h[i]);

printf("\n\nEnter received 7 bits (space separated): ");

for(i = 1; i <= 7; i++) scanf("%d", &h[i]);

c1 = h[1] ^ h[3] ^ h[5] ^ h[7];

c2 = h[2] ^ h[3] ^ h[6] ^ h[7];

c4 = h[4] ^ h[5] ^ h[6] ^ h[7];

pos = c4 * 4 + c2 * 2 + c1;

if(pos == 0)

printf("No error detected\n");

else{

printf("Error detected at position %d\n", pos);

h[pos] = !h[pos];

printf("Corrected code: ");

for(i = 1; i <= 7; i++) printf("%d", h[i]);

printf("\n");

}

return 0;

}

How to run: Input: 1 0 1 1 then 0 1 1 0 1 1 1 → code 0110011, error at position 5, corrected.

## Q5 – Flow control using Stop-and-Wait

File: q5_stopwait.c

// Q5: Flow control using Stop-and-Wait protocol

#include <stdio.h>

int main(){

int n, i, lostf, losta;

printf("Enter number of frames: ");

scanf("%d", &n);

printf("Enter frame number to lose (0 = none): ");

scanf("%d", &lostf);

printf("Enter ACK number to lose (0 = none): ");

scanf("%d", &losta);

for(i = 1; i <= n; i++){

printf("\nSender: Frame %d sent\n", i);

if(i == lostf){

printf("Frame %d LOST -> Timeout\n", i);

printf("Sender: Frame %d re-sent\n", i);

}

printf("Receiver: Frame %d received, ACK %d sent\n", i, i);

if(i == losta){

printf("ACK %d LOST -> Timeout\n", i);

printf("Sender: Frame %d re-sent\n", i);

printf("Receiver: Duplicate frame %d discarded, ACK %d sent again\n", i, i);

}

printf("Sender: ACK %d received\n", i);

}

printf("\nAll frames transmitted successfully.\n");

return 0;

}

How to run: Input: 3, 2, 3 (3 frames, frame 2 lost, ACK 3 lost).

## Q6 – Go-Back-N (corrupted frame, lost frame, lost ACK)

File: q6_gobackn.c

// Q6: Flow control using Go-Back-N (corrupted frame, lost frame, lost ACK)

#include <stdio.h>

int main(){

int n, w, ch, x, i, base = 1, end, done = 0;

printf("1. Corrupted frame\n2. Lost frame\n3. Lost ACK\nEnter choice: ");

scanf("%d", &ch);

printf("Enter number of frames: ");

scanf("%d", &n);

printf("Enter window size: ");

scanf("%d", &w);

printf("Enter the frame number with the problem: ");

scanf("%d", &x);

while(base <= n){

end = base + w - 1;

if(end > n) end = n;

printf("\nWindow [%d - %d]\n", base, end);

for(i = base; i <= end; i++)

printf("Sender: Frame %d sent\n", i);

if(!done && x >= base && x <= end){

done = 1;

for(i = base; i < x; i++)

printf("Receiver: Frame %d received, ACK %d sent\n", i, i);

if(ch == 3){

printf("Receiver: Frame %d received, but ACK %d LOST\n", x, x);

for(i = x + 1; i <= end; i++)

printf("Receiver: Frame %d received, ACK %d sent\n", i, i);

if(x < end){

printf("Sender: ACK %d received (cumulative) -> frame %d also confirmed, no resend\n", end, x);

base = end + 1;

} else {

printf("Sender: Timeout -> Frame %d re-sent\n", x);

printf("Receiver: Duplicate frame %d discarded, ACK %d sent again\n", x, x);

base = end + 1;

}

continue;

}

if(ch == 1) printf("Receiver: Frame %d CORRUPTED -> discarded, NAK %d sent\n", x, x);

if(ch == 2) printf("Frame %d LOST\n", x);

for(i = x + 1; i <= end; i++)

printf("Receiver: Frame %d discarded (out of order)\n", i);

printf("Sender: %s -> Go back to frame %d, resend all frames from %d\n",

ch == 1 ? "NAK received" : "Timeout", x, x);

base = x;

} else {

for(i = base; i <= end; i++)

printf("Receiver: Frame %d received, ACK %d sent\n", i, i);

base = end + 1;

}

}

printf("\nAll %d frames transmitted successfully.\n", n);

return 0;

}

How to run: Run 3 times with choice 1 / 2 / 3, then 5, 3, 2 each time.

## Q7 – Selective Repeat ARQ

File: q7_selective.c

// Q7: Flow control using Selective Repeat ARQ

#include <stdio.h>

int main(){

int n, w, x, i, base = 1, end, done = 0, hit;

printf("Enter number of frames: ");

scanf("%d", &n);

printf("Enter window size: ");

scanf("%d", &w);

printf("Enter the lost frame number: ");

scanf("%d", &x);

while(base <= n){

end = base + w - 1;

if(end > n) end = n;

hit = (!done && x >= base && x <= end);

printf("\nWindow [%d - %d]\n", base, end);

for(i = base; i <= end; i++){

printf("Sender: Frame %d sent\n", i);

if(hit && i == x)

printf("Frame %d LOST\n", i);

else if(hit && i > x)

printf("Receiver: Frame %d received and BUFFERED, ACK %d sent\n", i, i);

else

printf("Receiver: Frame %d received, ACK %d sent\n", i, i);

}

if(hit){

done = 1;

printf("Timeout for frame %d -> Sender re-sends ONLY frame %d\n", x, x);

printf("Receiver: Frame %d received, ACK %d sent -> frames %d to %d delivered in order\n",

x, x, x, end);

}

base = end + 1;

}

printf("\nAll %d frames transmitted successfully.\n", n);

return 0;

}

How to run: Input: 6, 3, 2.

# PART B – Packet Tracer (Q8–Q15)

# Q8 – IP addressing and default gateway

File: Q8_IP_Gateway.pkt

Build: 1 router (2911), 2 switches (2960), 3 PCs on each switch. Connect Router → Switch0 first (becomes g0/0), then Router → Switch1 (g0/1).

| Device | IP address | Subnet mask | Default gateway |
| --- | --- | --- | --- |
| Router g0/0 | 192.168.1.1 | 255.255.255.0 | — |
| Router g0/1 | 192.168.2.1 | 255.255.255.0 | — |
| PC0 / PC1 / PC2 | 192.168.1.2 / .3 / .4 | 255.255.255.0 | 192.168.1.1 |
| PC3 / PC4 / PC5 | 192.168.2.2 / .3 / .4 | 255.255.255.0 | 192.168.2.1 |

en

conf t

no ip domain-lookup

int g0/0

ip add 192.168.1.1 255.255.255.0

no sh

int g0/1

ip add 192.168.2.1 255.255.255.0

no sh

end

wr

Result: PC0 → PC3 and PC5 → PC1 = Successful.

# Common network for Q9, Q11, Q12, Q13

PC0, PC1 ─ Switch0 ─ Router0 ═══ Router1 ─ Switch1 ─ PC2, PC3

Cable order: Router0 → Switch0, Router1 → Switch1, then Router0 → Router1 (so the router link is g0/1 on both), then PCs → switches.

| Device | IP address | Gateway |
| --- | --- | --- |
| PC0 / PC1 | 192.168.1.2 / 192.168.1.3 | 192.168.1.1 |
| Router0 g0/0 (LAN) | 192.168.1.1 | — |
| Router0 g0/1 (to Router1) | 10.0.0.1 | — |
| Router1 g0/1 (to Router0) | 10.0.0.2 | — |
| Router1 g0/0 (LAN) | 192.168.2.1 | — |
| PC2 / PC3 | 192.168.2.2 / 192.168.2.3 | 192.168.2.1 |

Router0 base:

en

conf t

no ip domain-lookup

int g0/0

ip add 192.168.1.1 255.255.255.0

no sh

int g0/1

ip add 10.0.0.1 255.255.255.0

no sh

exit

Router1 base:

en

conf t

no ip domain-lookup

int g0/0

ip add 192.168.2.1 255.255.255.0

no sh

int g0/1

ip add 10.0.0.2 255.255.255.0

no sh

exit

# Q9 – RIP and OSPF

RIP file: Q9_RIP.pkt

OSPF file: Q9_OSPF.pkt

## RIP

Router0:

router rip

version 2

no auto-summary

network 192.168.1.0

network 10.0.0.0

end

wr

Router1:

router rip

version 2

no auto-summary

network 192.168.2.0

network 10.0.0.0

end

wr

Check: sh ip route → R 192.168.2.0 [120/1] via 10.0.0.2. PC0 → PC2 = Successful.

## OSPF

Router0:

router ospf 1

network 192.168.1.0 0.0.0.255 area 0

network 10.0.0.0 0.0.0.255 area 0

end

wr

Router1:

router ospf 1

network 192.168.2.0 0.0.0.255 area 0

network 10.0.0.0 0.0.0.255 area 0

end

wr

Check: log shows OSPF … LOADING to FULL; sh ip route → O 192.168.1.0 [110/2]. PC1 → PC3 = Successful.

Tip: to switch an existing file from RIP to OSPF: conf t → no router rip → then the OSPF lines.

# Q10 – SMTP and DNS

File: Q10_SMTP_DNS.pkt

Build: Server0 + Switch0 + PC0 + PC1, all on the switch. No router, no gateway.

| Device | IP address | DNS server |
| --- | --- | --- |
| Server0 | 192.168.1.10 | — |
| PC0 | 192.168.1.2 | 192.168.1.10 |
| PC1 | 192.168.1.3 | 192.168.1.10 |

- Server0 → Services → DNS → On. Add A Records: www.cisco.com → 192.168.1.10, mail.cisco.com → 192.168.1.10, pc1.cisco.com → 192.168.1.3.

- Server0 → Services → EMAIL → SMTP ON, POP3 ON → Domain cisco.com → Set → add users pc0 / 123 and pc1 / 123 with “+”.

- PC0 → Desktop → Email: name pc0, email pc0@cisco.com, incoming + outgoing mail.cisco.com, user pc0, password 123 → Save.

- PC1 → same with pc1 / pc1@cisco.com.

- PC0 → Compose → To pc1@cisco.com, Subject 24bce2079 → Send → “Send Success”.

- PC1 → Receive → “Receive Mail Success” and the mail appears.

# Q11 – Unicast routing with LSR

File: Q11_LSR_OSPF.pkt

LSR in Packet Tracer = OSPF. Use the common network + the OSPF commands from Q9.

Viva: Link-state: every router knows the whole map and runs Dijkstra’s shortest-path algorithm. Metric = cost (bandwidth).

# Q12 – Unicast routing with DVR

File: Q12_DVR_RIP.pkt

DVR in Packet Tracer = RIP. Use the common network + the RIP commands from Q9.

Viva: Distance-vector: routers share tables with neighbours every 30 s using Bellman-Ford. Metric = hop count, max 15. Problem: count-to-infinity (fixed by split horizon).

# Q13 – Unicast routing with BGP

File: Q13_BGP.pkt

Common network, no RIP/OSPF (if present: no router rip / no router ospf 1).

Router0 (AS 100):

conf t

router bgp 100

neighbor 10.0.0.2 remote-as 200

network 192.168.1.0 mask 255.255.255.0

end

wr

Router1 (AS 200):

conf t

router bgp 200

neighbor 10.0.0.1 remote-as 100

network 192.168.2.0 mask 255.255.255.0

end

wr

Check: “%BGP-5-ADJCHANGE: neighbor … Up”; press ⏩ ~8 times; sh ip route → B 192.168.1.0 [20/0]. PC0 → PC3 = Successful.

# Q14 – TCP and UDP traffic control

File: Q14_TCP_UDP.pkt

Code: Q14_code.py

Build: PC0 + PC1 + Switch0. PC0 = 192.168.1.1, PC1 = 192.168.1.2, mask 255.255.255.0.

- PC0 → Programming → New → name ex8, template Empty - Python3 → Create → open main.py → paste Q14_code.py → keep ROLE = "server" → Run.

- PC0 output: [TCP] server listening on port 5000: True and [UDP] server listening on port 6000: True.

- PC1 → same, but change line 11 to ROLE = "client" → Run. Wait ~40 seconds.

- Result: TCP rounds (slow start, flow control, congestion, Go-Back-N) and UDP retransmissions, both ending RESULT: SUCCESS - 100% delivered.

- When reopening the saved file: click Run on PC0 first, then on PC1.

Full Q14 code (same on both PCs, only line 11 changes):

# EX.NO 8 - TCP and UDP with Traffic Control Mechanisms (Cisco Packet Tracer)

# Same code on both PCs. Only change ROLE:

#   PC0 (192.168.1.1) -> ROLE = "server"   (run FIRST)

#   PC1 (192.168.1.2) -> ROLE = "client"

# TCP: token bucket (shaping) + sliding window (flow control) + slow start/AIMD (congestion control)

# UDP: leaky bucket (shaping) + Stop-and-Wait ARQ (reliability over a lossy network)

from tcp import *

from udp import *

from time import *

ROLE = "server"

SERVER_IP = "192.168.1.1"

TCP_PORT = 5000

UDP_PORT = 6000

UDP_CLIENT_PORT = 6001

MSG = "Hello this is 24BCE2079"

CHUNK = 4

RWND = 3                   # TCP receiver window (flow control)

TCP_DROP = [5]             # TCP segment 5 lost once (congestion)

UDP_DROP_DATA = [1, 2, 4, 4]

UDP_DROP_ACK = [4]

RATE = 5                   # token bucket: tokens per second

CAP = 3                    # token bucket size

INTERVAL = 0.5             # leaky bucket: 1 packet every 0.5 s

TIMEOUT = 2

MAX_RETRIES = 10

def split_msg():

chunks = []

for i in range(0, len(MSG), CHUNK):

chunks.append(MSG[i:i + CHUNK])

return chunks

# ======================= SERVER (PC0) =======================

tcp_server = TCPServer()

udp_sock = UDPSocket()

udp_st = {"expected": 0, "msg": "", "printed": False}

def onTCPNewClient(client):

st = {"expected": 0, "msg": "", "buf": ""}

def onChange(type):

print("[TCP] client " + client.remoteIP() + " state: " + str(type))

def handle(line):

if line == "FIN":

print("")

print("[TCP] Full message received: " + st["msg"])

client.send("DONE|" + st["msg"] + "\n")

return

parts = line.split("|", 2)

seq = int(parts[1])

if seq in TCP_DROP:

TCP_DROP.remove(seq)

reply = "NAK"

print("  seq=" + str(seq) + " LOST (simulated congestion) -> NAK")

elif seq == st["expected"]:

st["msg"] = st["msg"] + parts[2]

st["expected"] = st["expected"] + 1

reply = "ACK"

print("  seq=" + str(seq) + " accepted '" + parts[2] + "' -> ACK (rwnd=" + str(RWND) + ")")

else:

reply = "NAK"

print("  seq=" + str(seq) + " out of order, discarded -> NAK")

client.send(reply + "|" + str(seq) + "|" + str(RWND) + "\n")

def onReceive(data):

st["buf"] = st["buf"] + data

while "\n" in st["buf"]:

parts = st["buf"].split("\n", 1)

st["buf"] = parts[1]

handle(parts[0])

client.onConnectionChange(onChange)

client.onReceive(onReceive)

def onUDPServerReceive(ip, port, data):

if data == "START":

udp_st["expected"], udp_st["msg"], udp_st["printed"] = 0, "", False

print("")

print("[UDP] new session from " + ip + ":" + str(port))

udp_sock.send(ip, port, "START_ACK")

return

if data == "FIN":

if not udp_st["printed"]:

print("")

print("[UDP] Full message received: " + udp_st["msg"])

udp_st["printed"] = True

udp_sock.send(ip, port, "DONE|" + udp_st["msg"])

return

parts = data.split("|", 2)

seq = int(parts[1])

if seq in UDP_DROP_DATA:

UDP_DROP_DATA.remove(seq)

print("  seq=" + str(seq) + " DROPPED (simulated loss)")

return

if seq == udp_st["expected"]:

udp_st["msg"] = udp_st["msg"] + parts[2]

udp_st["expected"] = udp_st["expected"] + 1

print("  seq=" + str(seq) + " received '" + parts[2] + "'")

else:

print("  seq=" + str(seq) + " duplicate (ACK was lost), re-ACK")

if seq in UDP_DROP_ACK:

UDP_DROP_ACK.remove(seq)

print("  ACK " + str(seq) + " DROPPED (simulated loss)")

return

udp_sock.send(ip, port, "ACK|" + str(seq))

def run_server():

tcp_server.onNewClient(onTCPNewClient)

print("[TCP] server listening on port " + str(TCP_PORT) + ": " + str(tcp_server.listen(TCP_PORT)))

udp_sock.onReceive(onUDPServerReceive)

print("[UDP] server listening on port " + str(UDP_PORT) + ": " + str(udp_sock.begin(UDP_PORT)))

while True:

sleep(3600)

# ======================= CLIENT (PC1) =======================

tcp_client = TCPClient()

tcp_replies = []

udp_replies = []

cst = {"buf": "", "done": None, "tokens": CAP, "sent": 0, "retx": 0}

def onTCPClientReceive(data):

cst["buf"] = cst["buf"] + data

while "\n" in cst["buf"]:

parts = cst["buf"].split("\n", 1)

cst["buf"] = parts[1]

if parts[0].startswith("DONE|"):

cst["done"] = parts[0].split("|", 1)[1]

else:

tcp_replies.append(parts[0])

def onUDPClientReceive(ip, port, data):

udp_replies.append(data)

def tick():

sleep(0.1)

cst["tokens"] = min(CAP, cst["tokens"] + RATE * 0.1)

def consume_token():

while cst["tokens"] < 1:

print("    [token bucket] empty, waiting...")

tick()

tick()

cst["tokens"] = cst["tokens"] - 1

def show_result(received):

print("")

print("Sent     : " + MSG)

print("Received : " + str(received))

if received == MSG:

print("RESULT   : SUCCESS - 100% delivered")

else:

print("RESULT   : MISMATCH")

def run_tcp_client():

print("========== TCP ==========")

tcp_client.onReceive(onTCPClientReceive)

print("Connecting to " + SERVER_IP + ":" + str(TCP_PORT) + " -> " + str(tcp_client.connect(SERVER_IP, TCP_PORT)))

sleep(3)   # wait for the 3-way handshake

chunks = split_msg()

n = len(chunks)

print("Message split into " + str(n) + " segments")

cwnd, ssthresh, rwnd, base, rnd = 1, 8, 4, 0, 0

while base < n:

rnd = rnd + 1

win = min(cwnd, rwnd)

end = min(base + win, n)

note = ""

if rwnd < cwnd:

note = "  <- limited by receiver (flow control)"

print("")

print("Round " + str(rnd) + ": cwnd=" + str(cwnd) + " rwnd=" + str(rwnd) +

" ssthresh=" + str(ssthresh) + " -> window=" + str(win) + note)

for seq in range(base, end):

consume_token()

tcp_client.send("SEQ|" + str(seq) + "|" + chunks[seq] + "\n")

print("  sent seq=" + str(seq) + " data='" + chunks[seq] + "'")

t = 0

while len(tcp_replies) < end - base and t < 100:

tick()

t = t + 1

acked, lost = base, -1

for k in range(end - base):

if not tcp_replies:

break

parts = tcp_replies.pop(0).split("|")

seq = int(parts[1])

rwnd = int(parts[2])

if parts[0] == "ACK" and lost < 0:

acked = seq + 1

print("  ACK " + str(seq))

elif lost < 0:

lost = seq

print("  NAK " + str(seq) + " (packet lost)")

if acked < end and lost < 0:

lost = acked   # no reply in time = timeout

if lost >= 0:

ssthresh = max(cwnd // 2, 1)

cwnd = 1

print("  CONGESTION -> ssthresh=" + str(ssthresh) + ", cwnd=1, resend from seq " +

str(acked) + " (Go-Back-N)")

base = acked

else:

base = end

if cwnd < ssthresh:

cwnd = cwnd * 2

print("  slow start -> cwnd=" + str(cwnd))

else:

cwnd = cwnd + 1

print("  congestion avoidance -> cwnd=" + str(cwnd))

tcp_client.send("FIN\n")

t = 0

while cst["done"] is None and t < 100:

tick()

t = t + 1

show_result(cst["done"])

tcp_client.close()

def udp_send_reliable(sock, payload, want, prefix):

for attempt in range(1, MAX_RETRIES + 1):

sleep(INTERVAL)                       # leaky bucket: constant rate

sock.send(SERVER_IP, UDP_PORT, payload)

cst["sent"] = cst["sent"] + 1

if attempt > 1:

cst["retx"] = cst["retx"] + 1

t = 0

while t < TIMEOUT * 10:

while udp_replies:

r = udp_replies.pop(0)

if r == want or (prefix and r.startswith(want)):

return r

sleep(0.1)

t = t + 1

print("    timeout (attempt " + str(attempt) + ") -> retransmit")

print("No reply from server. Is the server running?")

return None

def run_udp_client():

print("")

print("========== UDP ==========")

sock = UDPSocket()

sock.onReceive(onUDPClientReceive)

print("UDP client on port " + str(UDP_CLIENT_PORT) + ": " + str(sock.begin(UDP_CLIENT_PORT)))

chunks = split_msg()

print("Sending " + str(len(chunks)) + " packets, leaky bucket = 1 packet / " + str(INTERVAL) + " s")

if udp_send_reliable(sock, "START", "START_ACK", False) is None:

return

for seq in range(len(chunks)):

print("  send seq=" + str(seq) + " data='" + chunks[seq] + "'")

if udp_send_reliable(sock, "SEQ|" + str(seq) + "|" + chunks[seq], "ACK|" + str(seq), False) is None:

return

print("  ACK " + str(seq))

reply = udp_send_reliable(sock, "FIN", "DONE|", True)

received = ""

if reply:

received = reply.split("|", 1)[1]

print("Packets  : " + str(len(chunks)) + " data, " + str(cst["sent"]) +

" transmissions, " + str(cst["retx"]) + " retransmissions")

show_result(received)

def main():

if ROLE == "server":

run_server()

else:

run_tcp_client()

sleep(2)

run_udp_client()

if __name__ == "__main__":

main()

Backup (no Programming tab): Server with HTTP + DNS, PC browses www.cisco.com in Simulation mode with filters TCP, UDP, DNS, HTTP → show the TCP 3-way handshake vs UDP with no handshake.

# Q15 – DNS client–server

File: Q15_DNS_Client_Server.pkt

Same build and IPs as Q10 (Server0 192.168.1.10, PC0 .2, PC1 .3, both DNS server 192.168.1.10). DNS records: www.cisco.com → 192.168.1.10, pc1.cisco.com → 192.168.1.3.

Test on PC0 → Command Prompt:

nslookup www.cisco.com     -> Address: 192.168.1.10

nslookup pc1.cisco.com     -> Address: 192.168.1.3

ping www.cisco.com         -> Reply from 192.168.1.10 (4/4, 0% loss)

Also: PC0 → Web Browser → www.cisco.com → page opens.

# If anything says Failed

- Red dot on a cable → go into that router port and type no sh.

- Check IP, mask, gateway (and DNS server for Q10/Q15) against the tables.

- Press ⏩ a few times and send the envelope again – the first one often fails.

- Router window won’t take typing → Window menu → pick the router; or close and reopen it.

- Still stuck → Simulation mode → Play ▶ – the ❌ shows exactly where it stops.